LIVE SCAN DATA TRANSMISSION SPECIFICATION

GEORGIA BUREAU OF INVESTIGATION

Date of Issue: 10/30/96

Revised 9/14/99

Revised 6/12/2001

Revised 1/31/2002

Revised 1/10/2005

Revised 2/4/2005

Revised 10/30/2006

Revised 3/29/2007

Revised 1/20/2010

Revised 1/21/2011

Revised 2/24/2011

Revised 5/2/2011

Revision Update

October 30, 2006 ……………………………. Changes to Section 3.4; 3.6.3.1;

3.6.3.2 and 3.6.3.3

October 30, 2006 ……………………………. NATMS – changed to NIST File

Collector (NFC)

March 29, 2007-------------------------------------- Changes to Section 3.4 (.tbl to .txt); 							      3.6.2 deleted last sentence.  3.6.3 							      added sentence.  3.6.3.1 Message 							      formats added Name (mrj), GBI 							      TCN and FBI TCN (mfi); deleted 							      SAN (mrj and mid).  3.6.3.2 replaced 						      all _ END _ with _ _ END _ _; added 						      TNETTCN to acknowledgment file.

August 1, 2007 -------------------------------------- Changes to Section 3.6.3.1POP 3

Mail Formats; added OTN to MID

message format.  Removed

<Attachment> from bottom of MFI

response message format.

January 20, 2010 ---------------------------------   Removed Appendices J, L, M, O, Q,

BB, CC, DD, EE, FF, GG, II, JJ, KK

January 20, 2010 ---------------------------------- Directed to Download from

valtabs.txt directory Appendices F, 							      J,L, M, N, O, P, Q, S, U, GG, II, JJ,

January 21, 2011 ___________________  Changes to Section 3.4 FTP   Common Partition Layout cnt.txt ; added information. 3.6.3.1 POP3 Mail Format; added FBI Name Search Message (MFN); MID (GBI) Ident response changed; FBI Name Search Response (MFN) and FBI Name Search Form added.

January 21, 2011 ------------------------------	Appendix V added COR and JCOR TOT

February 24, 2011 -----------------------------	Section 3.6.1 MID (FBI) Identification Response added “DHS Identification Response”.

May 2, 2011 -------------------------------------	CJIS Security Policy Version 5.0 Password Policy

CONTENTS

1.0	Introduction . . . . . . . . . . . . . . . . . . . . .  1.1

2.0 	Standards  . . . . . . . . . . . . . . . . . . . . . .   2.1

3.0	Live Scan Data

Transmission Specification. . . . . . . . .  3.1

4.0	Functional Specifications . . . . . . . . . .   4.1

5.0	Live Scan Workflow . . . . . . . . . . . . . . .  5.1

6.0	Network Interface Specification . . . . .  6.1

APPENDICES

Appendix A - Electronic Biometric Transmission Specification (www.fbibiospecs.org)

Appendix B - WAVELET Scaler Quantization Specification

Appendix C - American National Standard Institute Standard

www.itl.nist.gov/ANSIASD/sp500-245-a16.pdf)

Appendix D - Edit Criteria

Appendix E -	Arrest Disposition Number (ADN)

Removed 3/26/2007

Appendix F -	Translation Table for Caution

Download from valtabs.txt directory

Appendix G -	CCH Field Identifier Translation Table

Appendix H -	Court Disposition Numeric (CDN)

Removed 3/26/2007

Appendix I -	Court Provision Number (CPN)

Removed 3/26/2007

Appendix J -	Translation for Table for Eye Color

Removed 1/20/2010

Download from valtabs.txt directory

Appendix K -	Translation Table for General Offense Character (GOC)

Removed 3/26/2007

Appendix L -	Translation Table for Hair

Removed 1/20/2010

Download from valtabs.txt directory

Appendix M -	Translation Table for Miscellaneous Identifying Number

(MNU) Field

Removed 1/20/2010

Download from valtabs.txt directory

Appendix N -	Offense Code Listing

Download from valtabs.txt directory

Appendix O -	Translation Table for Race

Removed 1/20/2010

Download from valtabs.txt directory

Appendix P -	Translation Table for Reject Code

Download from valtabs.txt directory

Appendix Q -	Translation Table for Sex

Removed 1/20/2010

Download from valtabs.txt directory

Appendix R -	Translation Table for Skin

Removed 3/26/2007

Appendix S -	Scars, Marks & Tattoos (SMT) Code Listing

Download from valtabs.txt directory

Appendix T -	Supervision Status Number (SSN)

Removed 3/26/2007

Appendix U -	Translation Table For Turned Over To

Download from valtabs.txt directory

Appendix V -	Translation Table for TOT Type 1 Transactions

Appendix W-	Translation Table for TOT Type 1 Responses

Appendix X -	Administrative Message Code Table

Appendix Y - Identification Response Code Table

Appendix Z -  Acknowledgement Message Code Table

Appendix AA – NFS Protocol Version 3

Removed 1/31/2002

Appendix BB - Transmission Control Protocol

Removed 1/20/2010

Appendix CC - Internet Protocol Specification

Removed 1/20/2010

Appendix DD - Post Office Protocol Version 3

Removed 1/20/2010

Appendix EE - Network Processor & Multi-Protocol Network Program

2210 & 6611

Removed 1/20/2010

Appendix FF - Protocol Standards Defined by IEEE 802 & FDDI (c)

1992-1995 Microsoft Corp.

Removed 1/20/2010

Appendix GG - Ethernet Network

1994 Black Box Corp.

Removed 1/20/2010

Appendix HH – Translation Table for Purpose Code

Removed 2/4/2005

Download from valtabs.txt directory

Appendix II -    U. S. States and Territories

Removed 1/20/2010

Download from valtabs.txt directory

Appendix JJ - Foreign Countries

Removed 1/20/2010

Download from valtabs.txt directory

Appendix KK - Type 10 Facial Image Edit Criteria

Removed 1/20/2010

See EBTS Type 10 Facial and Type 15 Palm Print Record

Layout

1.0	INTRODUCTION

For nearly a century, fingerprinting has been the primary means of personal identification and during this period, the content, format, and quality of fingerprint cards has often been revised and refined. Yet, for the past fifty years, the basis for the exchange of fingerprint, identification, and arrest data between criminal justice agencies has been fingerprints captured using an ink process.  Within the last ten years, however, technology has provided a new fingerprint capture method to replace the inked card process.  This new technology enables live scan electronic capture of fingerprints.

The live scan system produces records by electronically scanning fingerprint images and capturing the images from live fingerprints.  This process requires no ink and uses advanced electronics to ensure accurate image capture for better quality images.  This technology has the potential to save time and effort at booking stations and to greatly improve the quality of post - arrest fingerprints.  Live scan can also permit the timely identification of individuals at booking stations and applicant processing.

Live scan systems also represent the major input component for the electronic submission of images to the state and national identification systems of the future.  The electronic interface allows a subject’s fingerprint images to be transmitted to other sites within minutes.  These electronic images are then received by a site’s AFIS (Automated Fingerprint Identification System) and an automatic search is initiated to determine proper identification.

The live scan fingerprint image capture process features a continuous real time image preview capability that allows the fingerprint image to be displayed on the monitor before, during and after capture, providing immediate verification of the quality of the fingerprint captured.  The operator rolls each finger on a flat lens platen for complete fingerprint image capture.  An on - screen display of the fingerprint capture sequence helps the operator roll the individual’s fingers in sequential order.

Previous live scan devices went directly from image capture to print processing.  Print process involved a “clean - up” of images prior to printing and certain data concerning ridge boundaries and pores were eliminated as part of this clean - up process.  Newer live scan models are designed to submit electronic images prior to print processing.  The shift towards direct transmission of the captured image prior to print processing, allows for a more complete fingerprint image since valuable fingerprint information such as ridge boundaries and pores are not eliminated as a result of the print process.

In the future, every image electronically submitted to the state must be compressed using the WAVELET Scaler Quantization Compression Algorithm.  This algorithm consists of a class of encoders for converting source fingerprint image data to compressed image data; a decoder process for converting compressed image data to reconstructed fingerprint image data; and coded representations for compressed image data. (Refer to WSQ specifications Appendix B)

The WSQ compression algorithm was selected for it’s compatibility with fingerprint data and GCIC will not accept uncompressed electronic images.  The compression rate for fingerprint images will be 15:1, however, GCIC will accept electronic images captured with a compression rate of 5:1 for those devices certified at the Minimum Image Quality Requirement level established by the FBI.

2.0	STANDARDS

Live scan systems must comply with the FBI’s Integrated Automated Fingerprint Identification System (IAFIS) Image Quality Specifications (IQS).  IAFIS is developed according to standards which establish an infrastructure for the exchange of fingerprint identification information between local, state and federal users, and between those users and the FBI.  To exchange fingerprint identification data effectively across jurisdictional lines or between dissimilar systems made by different manufactures, standards are needed to specify a common format for the data exchange. Therefore, live scan devices are required to meet the FBI’s IAFIS standards and those amended by GBI to include the following.

	The American National Standard Institute (ANSI/NIST) National Institute of Standards and Technology, Data Format for the Interchange of Fingerprint, Facial and other Biometric  Information (ANSI/NIST—ITL 1-2007)

This document provides guidelines for the exchange of biometric information between various federal, state, local tribal and international AFIS systems.

	Electronic Biometric Transmission Specification 	(EBTS), (IAFIS-DOC-01078-9.0 November 30, 2009)

This document specifies the file and record content, format, and data codes necessary for the exchange of 	fingerprint and biometric identification information between federal, local and state users and the FBI.  It provides a description of all requests and responses associated with electronic fingerprint identification services.  It also establishes error messages, specific compression algorithms for the exchange of fingerprint image information, and image quality assurance methods.

	The WAVELET Scaler Quantization (WSQ) Gray Scale

Fingerprint Image Compression Specification (IAFIS -

IC - 0110v2 February 16, 1993).

Specifies a class of encoders for converting source 				fingerprint image data to compressed image data; a decoder

process for converting compressed image data to

reconstructed fingerprint image data; and coded

representations for compressed image data.

- Georgia Bureau of Investigation Live Scan Data 				Transmission Specification (GBI June 1996).

This document contains specifications that govern how live scan

devices will electronically submit fingerprint and text data to the

Georgia Bureau of Investigation’s NIST File Collector (NFC).

- Criminal Justice Information Services Security Policy Version

5.0 Standard Authentication (Password)  (2/9/2011)

Agencies shall follow the secure password attributes, below, to authenticate an individual’s unique ID. Passwords shall:

1. Be a minimum length of eight (8) characters on all systems.

2. Not be a dictionary word or proper name.

3. Not be the same as the Userid.

4. Expire within a maximum of 90 calendar days.

5. Not be identical to the previous ten (10) passwords.

6. Not be transmitted in the clear outside the secure location.

7. Not be displayed when entered.

3.0	LIVE SCAN DATA TRANSMISSION SPECIFICATION

3.1	Scope

This section contains specifications that govern how live scan

devices will submit fingerprint and text data to the Georgia Bureau

of Investigation NIST File Collector (NFC).  The discussion is 			limited to software and procedural considerations.

3.2	Overview

Each live scan device will submit National Institute of Standards

and Technology (NIST) format text and images to the GBI NFC

using the File Transfer Protocol (FTP).  Before a live scan

device can connect to the NFC it will be provided with the

following items:

- Device ID

- Group ID

- Password

- TCP/IP address

- Name of the Host to access (must use Domain Name Service)

- A maximum transmit packet size

- The name of the “common” partition to retrieve its environment file from.

When these data items are obtained, the live scan device will use

the following procedure each time it initially connects to the

network:

- On initial connection, verify that it has the latest versions of validation tables by retrieving files found in the common partition.

- Update tables on the live scan that are out-of-date by retrieving updated files found in the common partition.

- Retrieve and read an “environment” file in the common partition that contains the name of a partition to which submissions will 	    be sent.

For each submission the live scan device will:

  Write the NIST data to the live scan device id directory.

  Write a signal file to a predefined directory.

After the NFC has recognized the submission and assigned a

control number to it, the control number will be made available to

the live scan device.

3.3	Data Transfer Using File Transfer Protocol (FTP)

Each live scan device submitting text and image data to the NFC will do so using the File Transfer Protocol across the Georgia Technology Authority (GTA) Router Network or via a secure encrypted Internet Protocol service.  It is important that the live scan device utilizes a version of FTP software that allows for passive transfers via a firewall or proxy.

The NFC will make two types of disk partitions available through the FTP mechanism.  The first type will be a common partition with read-only access.  It will contain the latest versions of GBI data validation tables and environmental information that will tell live scan devices which partition to write their submissions to.  The second type of partition will be used to receive submissions from the network.  There will be only one common partition.  There will be at least one submission partition. The number of submission partitions will depend on the workload.

Each live scan device will be assigned a unique three character

device identifier that must be used in all submissions to the

NFC.  The device will also be assigned to a logical group and

the group will be assigned to one of the FTP submission

partitions.  Each group will be assigned an unique four character

group identifier.

The NFC will not mount any resources shared by the live scan

device.  It is, therefore, the responsibility of the live scan device to

initiate all data transfers to the NFC and download all

validation tables from the NFC.  In general, data destined for

the NFC will be copied by the live scan device to the FTP

partition.

All text and image data submissions will be transmitted to the

shared partition in accordance with the specifications found in the

section “FTP Partition Layout”.

Copies of tables used by the live scan device to validate text data

prior to submission will be made available in a FTP partition.  It is

the responsibility of the live scan device to examine the directory

containing these tables and insure that the latest versions are

downloaded.  The live scan device will copy new tables from the

FTP partition as needed.  See “FTP Partition Layout” and

“Validation Revision Control File” for details.

3.4	FTP Common Partition Layout

The name of the common partition, which is read-only, will be /lscommon.

The common partition will contain a LS Configuration File for each group of live scan devices.  The LS Configuration File will contain one line of text that will be the name of the partition to which the devices in the group will write submissions.  All of these files will be kept in a directory called envfiles.  The name of the file will be the group id assigned to the live scan device.

For example, a live scan device assigned to group abc7 will retrieve and read the contents of the LS Configuration File called envfiles/abc7.  The first line of the file will contain, beginning in column one, the name of the partition to which the device will write its submissions.

Permissions on these files will be such that the live scan device will have access only to its own file.  The purpose of these environmental files is to allow the NFC system administrator to manage the distribution of disk space usage on the shared partitions.  The live scan device must read its environment file when it first connects to the network.  The live scan device administrator will be notified in advance of any changes to the mount points.

The environment file contains the following three elements:

- Path to partition

- Minimum check interval in minutes

- Maximum check interval in minutes

The check interval dictates the minimum and maximum frequency in which the submitting device should check the out directory and POP3 mailbox for new messages.

The environment file should be checked upon initial connection to the NFC.  If the device receives a shut down message, it should disconnect and recheck the environment file upon reconnect in case the information is changed between shutdown and restart.  If the NFC is shut down, the device should connect to check for a restart status no more often than the last minimum check interval it had retrieved for the NFC.  If the NFC still does not start, the device should disconnect again and check later.

The second directory will be used to distribute the latest versions of the data validation tables.  This directory will be called valtabs and will be at the same level as envfiles.

The valtabs directory will contain a copy of each validation table and a control file that will contain the implementation date of the latest version of each validation table.  The validation tables are to be downloaded daily at a minimum. The validation tables and the control file will all be stored as flat files with one line of text per entry.

The control file will be called revcon.dta.  It will contain a header record consisting of a eleven (11) character current table set version number which is the implementation date and time of the table set (always a past date).  The following lines will contain one line for each validation table.  Each line will start with an eleven (11) character table version number consisting of the year, date, and time of implementation (always a past date) of the table and the name of the table beginning in column twelve (12).  Each live scan device will be responsible for ensuring that it is using the latest version of each table. The year will be 0000-9999, date will be in Julian (001-366) format, time will be in hours and minutes based on a 24 hour clock.  The valtabs revcon.dta file should be checked whenever the device makes an initial connection and the NFC is found to be running.  It should then check no more often than the minimum check frequency and no less often than once a day.  If the revcon.dta file indicates that devices tables are out of date, that device should not submit a transaction until its tables are brought up to date.

In summary the common partition will be structured as follows:

/lscommon			start point of common partition

envfiles			directory containing assignment files

1a34				LS Configuration File for devices in

group 1a34

nc01				LS Configuration File for devices in

group nc01

more partition assignment files

valtabs			directory containing current validation

tables

revcon.dta			version control file

Example Tables

cnt.txt				contributing agency type table

NOTE:  The CNT valtabs table is changing its format.  The size of the agency name is changing from 30 characters to 45 characters.  The city and state will shift to the right.  The new format will be the following:

ORI = 9 characters

Agency Name = 45 characters

City = 30 characters

State = 2 characters

rfp.txt				applicant type table

sex .txt			sex type table

rac.txt				race type table

eye.txt				eye color table

hai.txt				hair color table

offense.txt			offense type table

smt .txt			scars, marks, tattoos type table

cau.txt				caution type table

reject.txt			reject type table

mnu .txt			miscellaneous number type table

pob.txt			place of birth type table

purpose.txt			purpose code type table

state.txt			state type table

citizen.txt			citizenship type table

tot .txt				turned over to type table

type1 .txt			type 1 table

type1res .txt			type 1 response type table

cch .txt			cch field identifier type table

remaining tables

3.5	FTP Submission Partition Layout

This section describes the 	layout of the submission partition.

In the event that the submitting device is a live scan store and forward concentrator with multiple live scan devices underneath, a group id will be assigned to that concentrator.  The concentrator’s individual live scan devices will be the only device subdirectories contained within that group id.  Submitting of NIST text and image data will be the same as below, noting that the live scan store and forward must write the data to the proper live scan device subdirectory associated with the submission.

Each live scan device will be assigned to a group and each group will be assigned to each subdirectory on one of the FTP partitions dedicated to the live scan network.  Each device in the group will be further assigned to a subdirectory beneath the group directory.  From the NFC’s point of view, the name of the subdirectory will  contain the name of the mount point of the partition, the group identifier and the identifier assigned to the live scan device.  For example, if the mount point is /ls1 and the device id is abc and the device assigned to group def3 then the name of the subdirectory will be

/ls1/def3/abc

This directory will be used to receive the NIST text and image data

from the live scan device.

Signal files will be sent to indicate that a submission is ready for

processing (See “Submitting Text and Images”).  There will be one

directory in each FTP partition to receive these signal files.  This

directory will be called sigfiles.  The sigfile directory will be used

by all devices assigned to that partition.

For example, using the mount point ls1 relevant directories would

be:

/ls1/def3/abc		NIST text and image directory assigned to live

scan device abc group def3.

/ls1/def3/123		NIST text and image directory assigned to live

scan device 123 in group def3.

/ls1/rst7/jkl		NIST text and image directory assigned to live

scan device jkl in group rst7.

/ls1/sigfiles		directory where all live scan devices assigned

to partition ls1 will write signal files.

3.6	Submitting Text and Images

As part of the submission process, the live scan will check the out

directories for the presence of a file named SUBMIT.  The

presence of this file indicates the live scan may submit NIST

records to NFC.  The absence of this file indicates that live scan

may not submit NIST records.  If live scan submits NIST records

without the SUBMIT file present, the submissions will 	be ignored

by NFC.

When the SUBMIT file is detected, the live scan device will copy

the text and image set to the NFC in the FTP submission

partition.  The text and image set will be concatenated into a single

file.  The file name of the text and image set must be in the format

of ddhhmmss to distinguish it from previously submitted records.

After transmission of text and images is complete, the live scan

device will copy a signal file to the partition’s sigfiles directory.

Once NFC has received a transaction it will write an

acknowledgement to the out file indicting the record has been

received.  Live scan will check the out file at a constant interval for

an acknowledgment that the transaction has been received by

NFC.  If the acknowledgment does not appear in the out file

after a maximum amount of time, the live scan will begin the

submission process again with the SUBMIT check and the

generation of a new ddhhmmss file.  It will be the responsibility of

NFC to process and clean up any of these “orphaned” sigfiles

and submission files.  The default values will be one minute

between checks and ten minutes total.  All text and image data will

be submitted in NIST format.

3.6.1	Text and Image File Name Set

At the time of submission, the device will create within its assigned

device directory a NIST file using a name composed of the date

and time of the submission.  The format will be:

ddhhmmss

where

dd    	is the current day of the month,

hh 	is the hour based on the 24-hour clock,

mm	is the minutes and

ss	is the seconds.

3.6.2	Signal File Format

When transmission of the text and image set is complete, the

device will create a signal file in the /sigfiles directory containing

one character of data using a name composed of the time stamp of

the submission and the live scan device id.  The name of the signal

file will be in the following format:

ddhhmmss.LLL

where

ddhhmmss	is the name of the submission file and

LLL		is the live scan device identifier.

3.6.3	Message Formats

Submission messages will be placed in the live scan “out” directory to be retrieved by the live scan using FTP as described in 3.6.3.2.  All other messages will be sent to live scan devices by POP3 mail message.

3.6.3.1   POP 3 Mail Formats

All NFC to live scan mail messages will contain a header record

composed of the following:

TO, FROM, SUBJECT, DATE/TIME

and a message record.  The message record will contain

identifying data items and possible “attachment” records.  At the

present time GBI has identified the following message record 			formats for POP 3 mail.

	REJECT   (mrj)

	IDENTIFICATION   (mid)

	FBI IDENTIFICATION   (mfi)

  	FBI NAME SEARCH    (mfn)

Provided below are the POP 3 record layouts for both the header 			and message record formats:

  HEADER RECORD FORMAT

TO:

FROM:   tnetsystem@TNET02.GBITNET.local

SUBJECT:   mrj, mid,  mfi or mfn

DATE/TIME:   YYYY/MM/DD   HH:MM:SS

  REJECT MESSAGE  FORMAT

TYPE:   	mrj

LSTCN:	Live scan TCN number

GBITCN:	TCN assigned by TNET subsystem

DATE/TIME:	YYYY/MM/DD   HH:MM:SS

NAME:

RCODE:	Return code numeric

RLITERAL:  Translation

NOTE:	Instructions from AFIS

  GBI IDENTIFICATION RESPONSE FORMAT

TYPE:   	mid

LSTCN	Live scan TCN number

GBITCN:   	TCN assigned by TNET subsystem

DATE/TIME:  YYYY/MM/DD   HH:MM:SS

NAME:

SID:    	State Identification number assigned by GBI

OTN:

OCA:

IDENT:	Record on File

No Prior Criminal History information is available for this request.

  FBI IDENTIFICATION RESPONSE FORMAT

TYPE:   	mfi – 	FBI Identification Response or

DHS Identification Response

LS TCN:   	Live Scan TCN number

GBI TCN:

DATE/TIME:  YYYY/MM/DD   HH:MM:SS

OCA:   	Originating Agency Case number

FBI NUMBER:   Federal Identification number assigned by

FBI

SID:   		State Identification number assigned by GBI

NAME:  	Name identified from FBI master record

FBI IDENT:	Y or N

NOTE:  When an Applicant (APP) transaction is rejected from the FBI twice a pre-populated form will be sent back to the submitting agency via live scan.  The agency staff will print out the form and manually submit to the FBI for a name search to be conducted.

Therefore, a new FBI Reject message has been created to provide the agency the name search form. (See below)

  FBI NAME SEARCH RESPONSE FORMAT

TYPE:   	mfn

LS TCN:   	Live Scan TCN number

GBI TCN:

DATE/TIME:  YYYY/MM/DD   HH:MM:SS

NAME:  	Sent  at time of request

FBI Name Search form below:

FBI CJIS NAME SEARCH REQUEST FORM

Please complete the form below to request a FBI name check. Please be advised that an individual’s fingerprints must be rejected twice for image quality prior to requesting a FBI name check.

ORI of State/Federal/Regulatory Agency: __________________________________________

Your Agency’s Point of Contact (POC) for the Response: _____________________________

Phone Number of POC: ________________________________________________________

FAX Number of POC: _________________________________________________________

Address of Requesting Agency: _________________________________________________

_________________________________________________

_________________________________________________

Please FAX _____ or mail _____ my response to this request.

______________________________________________________________________________

SUBJECT OF NAME CHECK

Transaction Control Number (TCN) of Subject’s Fingerprint Submission: __________________

Transaction Control Number (TCN) of Subject’s Fingerprint Submission: __________________

Name: ______________________________ Alias: _________________________________

Date of Birth: ________________________ Place of Birth: ___________________________

Sex: _____ Race: _____ Height: _____ Weight: _____ Eyes: _____ Hair: _________

Social Security Number: ___________________ Miscellaneous Number: _______________

State Identification Number: ___________________ OCA: __________________________

** Please note that highlighted fields are required for name check searches. **

Be sure to include the TCN from both rejected transactions.

FBI CJIS Division

ATTN: Name Check Request

1000 Custer Hollow Road

Clarksburg, WV 26306

FAX 304-625-5102

3.6.3.2   FTP Submission Record Formats

Provided below are the record layouts for the FTP record

formats.  The data elements may be in any order with the

exception of the  _ _ END_ _ line, which must be last.  The device 			may send additional information in the sigfile besides SIGF:.  Any 			additional information sent is not used by AFIS but will be echoed 			back with the acknowledgement and mac messages.  Any 				additional information must appear before_  _END_ _ .  Live scan 			will not honor any out file that does not have the_ _END_ _ marker 			present.

SIGNAL FILE

SIGF:		ddhhmmss.LLL

_ _END_ _

Where

SIGF		is the signal file name

_ _END_ _	is the end of file marker

ACKNOWLEDGMENT FILE

SIGF:		ddhhmmss.LLL

TNETTCN:

ARV:		ddhhmmss

SAN:		nnnnnnnnnn

_ _END_ _

Where

SIGF 		is the signal file name

TNETTCN:

ARV 		is the arrival time (ddhhmmss)

SAN 		is a unique NFC assigned acknowledgment

number.

_ _END_ _ 	is the end of file marker

3.6.3.3	FTP Message Formats

When NFC completes processing of a NIST record or has a

notification for the live scan device(s), a message file named

mxxddhhmmss.LLL will be written into the out directory detailing 			the results.  mxxddhhmmss.LLL refers to the following:

m	general file description

m indicates a message file

xx	specific file description

ac indicates a acknowledgment message

ad indicates an administrative message

ddhhmmss	date and time message file was written to the

out directory by NFC

LLL		live scan ID

There are currently two valid file types:

ACKNOWLEDGEMENT	(ac)		macddhhmmss.LLL

ADMINISTRATIVE		(ad)		maddhhmmss.LLL

The live scan device will find these messages in the

out directory in the form of mxxddhhmmss.LLL.

The content of the message file will be identical to the initial out file 			(minus the END marker) with the following data elements added: 			RC, REG, TCN, MSG,_ _END_._  The data elements, except for 			the MSG line, may be in any order with the exception of the 				_ _END_ _ line which must be last.  Live scan will not honor the 			message file 	unless the _ _END_ _ marker is present.  The live 			scan device will delete each POP3 message and FTP file from the

server after reading.  (See Appendices X, Y & Z for Message

Codes)

mac

SIGF:		ddhhmmss.LLL

TNETTCN:

ARV:		is the arrival time ddhhmmss

SAN:		unique NFC assigned number

RC:		code(s) from reject table

REG:		time quick edit checks performed

NCN:		internal order number

_ _END_ _	end-of-file marker

mad

RC:		code from administrative table

MSG:		description of administrative code

_ _END_ _	end-of-file marker

3.6.3.4	FLOW CONTROL

Flow control message files are intended to control the live scan

NIST submission data flow.  These messages will be delivered in

the out directory detailing the condition in the form

mxxddhhmmss.LLL.  Currently valid file descriptions will be ‘mad’ 		which indicates an administrative notification from NFC to a

specific live scan or to all live scan devices.  This ‘mad’ message

has a higher priority in processing than the ‘mrj’ or ‘mid’.  When

the out SUBMIT file status is modified, a flow control message

detailing the status change will be written detailing the reason for

the SUBMIT change.  The format of the flow control files will be:

RC,  MSG,_  _END_._ Live scan will ignore the flow 	control

messages if the _ _END_ _ marker is not present.  Live scan will

delete the messages after reading.

The reactions of the live scan devices for each type of flow control

message are defined in the following table:

| NFC Message | NFC Situation | Live Scan Will | Return to Normal Message Expected | Live Scan Will Then |
| --- | --- | --- | --- | --- |
| ‘mad’ : A50 or A20 out/SUBMIT file removed. | NFC queue is full. No more NIST submissions can be received. | Stop Transmissions | ‘mad’ : A00 or out/SUBMIT file added. | Resume normal transmissions |
| ‘mad’ : A10 | NFC queue is approaching full. | Slow down transmissions. Live scan will increase its out file check interval. | ‘mad’: A00 | Resume normal transmission rate |
| ‘mad’: A90 out/SUBMIT file removed | NFC is shutting down | Stop transmissions | ‘mad’: A00 or out/SUBMIT file added | Resume normal transmissions |

3.7	Directory Schematic

The general structure of the FTP partition in diagram form appears

below.

ls1			partition start point

sigfiles		directory for signal files from all devices

assigned to this partition

nc2a			directory for group nc2a (individually

connected livescan devices)

478			directory for live scan device 478

ddhhmmss		text and images for one submission

ddhhmmss		text and images for one submission

def7			directory for group def7 (livescan store and

forward concentrator)

abc			directory for live scan device abc

ddhhmmss		text and images for one submission

ddhhmmss		text and images for one submission

123			directory for live scan device 123

ddhhmmss		text and images for one submission

ddhhmmss		text and images for one submission

3.8	File Definitions

Listed below are the various definitions of files that will interact with

the live scan device.  The order listed is the order encountered.

3.8.1	LS Configuration File

The LS Configuration File is a control file that contains data items

that the live scan device needs to complete the connection to the

NFC.  This file will contain the name of the partition a live scan

device will write its submissions to.  Also contained will be

maximum timer values for the purpose of checking for

administrative messages from the NFC.  The LS configuration

file will contain only a single line with the data fields separated by

commas and terminated with a new line character.  There will be

one file for each livescan group.

File name	-	/(lscommon)/envfiles/(livescan group id)

Contents	-	Sigfiles Partition, NIST Data Partition, 							Minimum Timer(minutes), Maximum 							Timer (minutes)

Sample file	-	/ls1/sigfiles, /ls1/gggg,1,10

3.8.2	Validation Revision Control File

The Validation Revision Control File contains the version number

and implementation date of each validation table in the valtabs

directory.  As validation tables are to be updated, the date at which

the file was replaced in the /(common partition)/valtabs directory

will be entered into this file. The control file will be called

revcon.dta.  It will contain a header record consisting of the

implementation date and time of the table set (always a past date).

The following lines will contain one line for each validation table.

Each line will start with a table version number consisting of the

year (yyyy), date (Julian), and time (hhmm) of implementation

(always a past date) and the name of the table beginning in column

twelve (12).  Each live scan device will be responsible for ensuring

that it is using the latest version of each table.

File name	-	/(lscommon)/valtabs/revcon.dta

Contents	-	Implement Date Time(yyyyjjjhhmm) 

Date Time(yyyyjjjhhmm) table name

Sample file	-	19943321201

19942551201 agency.txt

19933310001 atyp.txt

19941821201 booking.txt

3.8.3	New Submission Signal File to NFC

When transmission to the NFC of the text and image set is

complete, the device will create a signal file containing one

character of data using a name composed of the time stamp of the

submission and the live scan device id.  The name of the signal file 		will be in the following format:

ddhhmmss.LLL

Where

ddhhmmss is the name of the submission file

LLL is the live scan device identifier.

The actual contents of this signal file will be ignored.  Only the

name is important.

File name	-	/(partition)/sigfiles/(timestamp.livescan id)

Contents	-	one character

Sample Name -	/ls1/sigfiles/29134327.478

Sample File	-  	

3.8.4	Re-Submission of Error Transactions

When the transmission of the text and image set is complete and errors have been detected an “ mrj”  message is returned to the submitting live scan device.  The following procedures are to be performed:

- Correct the error(s)

- Use the same TCN number

- Create a new sigfile

- Resubmit transaction to NFC

3.8.5	Live Scan TCN Format

Listed below is the format for the live scan TCN number.  This number consists of the following:

Live scan ID = 3 character field

Last number of the current year = 1 character field

Unique generated live scan number = 6 character field

Sample format:

0121000123

012	=	LS ID

1	=	last number of current year

000123 =	unique generated live scan number

4.0	FUNCTIONAL SPECIFICATIONS

The following sections are designed to clarify basic requirements

of the Live Scan function.  An exhaustive listing of detailed

requirements and functions is not provided and is beyond the

scope of this section.

4.1	ELECTRONIC SUBMISSION - BOOKING PROCESS

LOCAL AGENCY

1.0	Operator Logon

1.0.1	Enter operator id

1.0.2	Enter password

1.0.3	Logon correct?

= Yes GOTO 1.1

= No   GOTO 1.0

1.1	Check NFC common partition for LS Configuration file

Check NFC common partition for Valtabs directory

1.1.1	Live scan revcon number = NFC revcon.dta file?

= Yes GOTO 1.2

= No   Live scan checks NFC valtab tables

If NFC valtabs does not equal live scan

valtabs

Download valtabs

1.1.2	Last table in valtab directory?

= Yes	GOTO 1.2

= No	GOTO 1.1.1

1.2	Operator enters arrest and demographic data at live scan,

booking or case management system.

1.3	Data edits are performed on entered data.

1.3.1	Data edits correct?

= Yes	GOTO 1.4

= No	GOTO 1.2

1.4	Data sent to (LIVE SCAN FINGERPRINT QUEUE)

1.5	Select card type (criminal,applicant)

1.6	Select ORI/OAC number

1.7	Select subject’s name from list or enter search parameters.

1.8	Demographic data screen appears with subject’s

information.

1.8.1	Operator verifies data matches with subject.

1.8.2	Data OK?

= Yes	GOTO 1.9

= No	GOTO 1.7

1.9	Fingerprint screen appears

1.9.1	Roll finger

1.9.2	Fingerprints acceptable?

= Yes GOTO 1.9.3

= No   GOTO 1.9.1

1.9.3	Last finger?

= Yes GOTO 1.10

= No   GOTO 1.9.1

1.10	Check charges for GBI criterion offenses

1.10.1	GBI submission?

= Yes Print card for local agency

GOTO 1.11

= No  Print card for local agency

END booking process

1.11	Demographic and arrest data reformatted into type 1 and

type 2 records

1.12	Fingerprint data formatted into type 4 record using WSQ

compression algorithm.

1.13	Live scan checks NFC out directory for

SUBMIT file.

1.13.1	SUBMIT file present?

= Yes GOTO 1.14

= NO  GOTO 1.13

1.14	Live scan sends record to gggg/LLL group id

1.15	Live scan writes to signal files.

1.16	Live scan checks out file

1.16.1	Out file present?

= Yes live scan deletes out file record

= No  live scan increments check count

1.16.2	Check count = 10?

= Yes GOTO 1.13

= No  GOTO 1.16

4.2	MESSAGE HANDLING

2.1	Operator reviews message

2.2 	Print screen?

= Yes Message information sent to print server.

GOTO 2.3

= No  GOTO 2.3

2.3	Operator clears message

2.4	Message type = Reject?

= Yes Select subject from list or enter search

parameter

GOTO 2.5

= No   End

2.5	Fingerprint error?

= Yes Operator re-rolls requested fingerprints

= No   GOTO 2.6

2.6	Demographic error?

= Yes Operator corrects entry at live scan.

GOTO 2.7

= No  GOTO 2.7

2.7	NIST error?

= Yes resend transaction

GOTO 2.8

= No  GOTO 2.8

2.8	Demographic, arrest and fingerprint data formatted

into Type 1, Type 2 and Type 4 records.

2.9	Type 1, Type 2 and Type 4 records sent to agency’s

NFC submission partition for device.

5.0	LIVE SCAN WORKFLOW

This section represents the Live Scan system workflow.

6.0     NETWORK INTERFACE SPECIFICATION

6.1     INTRODUCTION

Law enforcement agencies throughout the state of Georgia will 			soon be able to take advantage of Georgia’s upgraded Automated 			Fingerprint Identification System (AFIS) by installing new live scan 			devices or upgrading current live scan systems.

Benefits from the new live scan device include better quality 				fingerprints for tenprint and latent searching, faster processing and 			response for 	booking of offenders and improved identification 			capabilities throughout the state.  Installation of the enhanced live 			scan devices will coincide with the upgrade of the State of 				Georgia’s AFIS.

Current live scan devices are not equipped to support the format 			and electronic transmission of fingerprint images, demographic 			data or arrest data as defined in the National Standard for 				Fingerprint Image Transmission.

The updated live scan device will utilize the Georgia Technology Authority (GTA) Network and/or via the Internet for communications between the local law enforcement agency and the GBI’s NIST File Collector (NFC).  The NFC is the component of the AFIS that stores, processes and routes fingerprint image data, criminal history update information and search results.

The Network Interface Specification uses widely available Internet 		protocols.  Most of the protocols mentioned in this document are 		used in open systems. The FTP protocol is designed to be 			portable across different hardware platforms, operating systems, 		network architectures and allows for passive transfers via a firewall

or proxy.  The Transmission Control Protocol (TCP) is intended for use as a highly reliable host-to-host protocol.  The Internet Protocol (IP) provides for transmitting blocks of data called data grams from sources to destinations, where sources and destinations are hosts identified by fixed length addresses.

The Internet protocol also provides for fragmentation and 				reassembly of long datagrams.  The Post Office Protocol - Version 			3 (POP3) is intended to permit a workstation to dynamically access 		a maildrop on a server host.  POP3 protocol allows a workstation to 		retrieve mail that the server is holding for it.

The wide spread availability of software and hardware supporting 			the above protocols should ease the Live Scan vendor’s and local 			agencies’ burden in establishing a connection to the Georgia AFIS.

6.2     Types of Protocol

- FTP

- TCP

- IP

- POP3

6.3   LAN Connection

IEEE 802 LANs

- 802.2 Ethernet (Common To All)

- 802.3 (CSMA)

6.4     Network Cabling

- 10 BASE 2

- 10 BASE T

6.5     Transmission

- 56KB (minimum)

6.6    Routers

- 6611 IBM

- 2210 IBM

6.7    Network Interface Workflow

This section represents the Network Interface workflow.

APPENDIX D

Please refer to EBTS Type 1 and Type 2 Record layouts for the maximum number of BYTES including character separators and field number for each.

APPENDIX E

APPENDIX F

APPENDIX G

APPENDIX H

APPENDIX I

APPENDIX J

APPENDIX K

APPENDIX L

APPENDIX M

APPENDIX N

APPENDIX O

APPENDIX P

APPENDIX Q

APPENDIX R

APPENDIX S

APPENDIX T

APPENDIX U

APPENDIX V

TRANSLATION TABLE FOR TOT TYPE 1 TRANSACTIONS

CAR		Criminal Ten-Print Submission (Answer Required)

CNA		Criminal Ten-Print Submission (No Answer 					Required)

JUV		Juvenile

APP		Applicant Submission

SOT		Search Only

COR		Corrections

JCOR	Juvenile Corrections

APPENDIX W

TRANSLATION TABLE FOR TOT TYPE 1 RESPONSES

SRE			Submission Results - Electronic

ERRT		Transaction Error

APPENDIX X

ADMINISTRATIVE MESSAGE CODE TABLE

Type		Code

mad		A000:	Resume transmission

A010:	Reaching input limit

A020:	Stop transmission

A050:	NFC queue is full

A090:	NFC shutting down

APPENDIX Y

Offender Tracking Number (OTN) and State

Identification number (SID) Format

OTN is an numeric 11 character field with the 11th character being a check character.  The 11th character is the remainder after dividing the first 10 characters by 7.

Sample format:

46385835 = 4638583 divided by 7 leaves a remainder of 5 (the last character).

SID is an alphanumeric 8 character field.  SID numbers less than 1075000 must be numeric right justified, zero filled.  All SID numbers greater than 1074999 require a check character may be alphabetic.  If the 8th character is alphabetic, a check character must be determined as follows:

Multiply position	1	by	2	giving		A

2	by	7	giving		B

3	by	6	giving		C

4	by	5	giving		D

5	by	4	giving		E

6	by	3	giving		F

7	by	2	giving		G

To determine the check character add 39 + A + B + C + D + E + F + G = SUM.

Divide the SUM by 11 giving a remainder.

The remainder corresponds to the following check characters:

Remainder:	0  1   2   3  4  5  6  7   8  9  10

Character:	A  E  H  J  K  L  M  P  T  W  X

Example:	SID / 2090187P

2 * 2 = 4

0 * 7 = 0

9 * 6 = 54

0 * 5 = 0

1 * 4 = 4

8 * 3 = 24

7 * 2 = 14

100 + 39 = 139/11= 12 remainder 7 = P

APPENDIX Z

ACKNOWLEDGEMENT MESSAGE CODE TABLE

Type			Code

mac			0000	Positive

0001	Unreadable/not accepted

(list may be expanded)

APPENDIX HH

APPENDIX II

APPENDIX JJ

APPENDIX KK

TYPE 10 FACIAL and TYPE 15 PALM PRINT IMAGE SPECIFICATIONS

Please refer to;

Type 10 Facial and Type 15 Palm Print Record Layout; NIST Special Publication 500-271; ANSI/NIST – ITL 1-2007, American National Standard for Information Systems, Data Format for the Interchange of Fingerprint Facial and Other Biometric Information Part 1; National Institute of Standards and Technology

Criminal Justice Information Services (CJIS) Electronic Biometric Transmission Specification (EBTS) IAFIS-DOC-01078-9.0 (November 2009)